# PV102 Day 6 Student Packet

Inverter Charge Controller and Protection Boundaries

PV102 | CTS Day 22 | Student Print Packet
Power electronics set system boundaries. | Tuesday, August 11, 2026

# Today's rule

Power electronics set system boundaries.

# Learning targets

Explain the roles of charge controllers and inverters in battery-based PV systems.

Match inverter selection to continuous load, surge load, voltage, and output requirement.

Identify where disconnects and overcurrent protection belong conceptually.

Create a block diagram that separates source, storage, conversion, and load boundaries.

# Morning quiz

1. An inverter primarily converts:

A. DC to AC

B. AC to sunlight

C. wood to wire

D. shade to volts

2. A charge controller protects and manages:

A. Battery charging from PV

B. Roof pitch

C. Clipboards

D. Nameplate stickers only

3. Inverter sizing must consider:

A. Continuous load and surge

B. Marker color only

C. Roof direction only

D. Lunch menu

4. An overcurrent device is used to:

A. Open a circuit during excessive current

B. Store solar energy

C. Increase sunshine

D. Convert AC to shade

5. A disconnect provides:

A. A means to isolate equipment

B. Free battery capacity

C. More roof area

D. A better compass

6. Why should source, storage, conversion, and load boundaries be shown separately?

________________________________________________________________________________________________

________________________________________________________________________________________________

7. Name one label that belongs on a training system handoff diagram.

________________________________________________________________________________________________

________________________________________________________________________________________________

8. Why can an inverter with enough continuous watts still fail a job?

________________________________________________________________________________________________

________________________________________________________________________________________________

9. What is one thing students should not decide without manufacturer instructions?

________________________________________________________________________________________________

________________________________________________________________________________________________

10. Write the Day 6 rule in your own words.

________________________________________________________________________________________________

________________________________________________________________________________________________

# Reference: decision order

| Step | Action | Proof question |
| --- | --- | --- |
| 1 | Define the job | What must the system accomplish? |
| 2 | Collect evidence | What is measured, observed, or still unknown? |
| 3 | Choose boundary | What must be protected, isolated, sized, or verified? |
| 4 | Defend the choice | What evidence supports the decision? |
| 5 | Handoff | What should the next person know? |

# Balance-of-System Block Diagram

Teams build a labeled block diagram and defend every protection/isolation point.

## Scenario options

12 V training system with PV module, controller, LiFePO4 battery, inverter, DC load, AC load.

48 V bus system concept with hybrid inverter/charger, battery bank, PV strings, AC distribution.

Remote DC-only sensor system with no inverter and limited service visits.

## Team procedure

Choose a scenario and place component cards in energy-path order.

Draw source, storage, conversion, and load boundaries.

Add conceptual OCPD/disconnect labels where energy could feed a fault or where service isolation is needed.

Mark manufacturer-instruction-required decisions.

Run a peer safety check: can someone tell what is energized, isolated, and unknown?

Handoff the diagram and unresolved decisions.

## Team worksheet

| Prompt | Team response |
| --- | --- |
| System/job statement |  |
| Evidence used |  |
| Decision made |  |
| Safety boundary |  |
| Top unknowns |  |
| Next action |  |

# Exit ticket

Which boundary on your diagram mattered most for safety, and what would happen if it were missing?

________________________________________________________________________________________________

________________________________________________________________________________________________