# PV101 Day 7 Student Worksheet

Load assessment and basic system sizing | Guided practice

Day_07_PV101_Load_Assessment_and_System_Sizing_2026-07-15

Name: __________________________________  Date: __________________

# Operating rules

Use only the values provided.

Show formulas, numbers, and units.

Circle assumptions.

Check reasonableness before asking for signoff.

This is guided practice—not a product design or proposal.

# Provided training assumptions

| Item | Training value |
| --- | --- |
| System voltage | 12 V |
| Usable battery fraction / maximum depth of discharge | 80% for this exercise |
| Autonomy for this exercise | 1 day |
| Peak sun hours for this exercise | 4 h/day |
| Loss multiplier for this exercise | 1.25 |
| Inverter headroom for this exercise | 25% above concurrent AC load |

# Part A: RV training loads

| Load | Power | Hours/day | Calculation | Wh/day |
| --- | --- | --- | --- | --- |
| 3 interior LED lights | 8 W each | 4 h |  |  |
| Water pump | 60 W | 0.5 h |  |  |
| Vent fan | 36 W | 3 h |  |  |
| Phone/USB charging | 24 W | 2 h |  |  |
| Laptop charger (AC) | 65 W | 2 h |  |  |

Total daily energy: __________________ Wh/day

# Part B: Storage

Required stored Wh = daily Wh x autonomy = __________________________________
__________________________________________________________________________________

Battery nameplate Wh = required stored Wh / usable fraction = ________________________
__________________________________________________________________________________

Battery Ah at 12 V = battery nameplate Wh / 12 V = _________________________________
__________________________________________________________________________________

Reasonableness sentence: __________________________________________________________
__________________________________________________________________________________

# Part C: Array and inverter

Raw array watts = daily Wh / peak sun hours = __________________ W
__________________________________________________________________________________

Adjusted array watts = raw array watts x 1.25 = __________________ W
__________________________________________________________________________________

Concurrent AC load used for inverter sizing: __________________ W
__________________________________________________________________________________

Minimum inverter rating with 25% headroom: __________________ W
__________________________________________________________________________________

What load or assumption most affects the result? _____________________________________
__________________________________________________________________________________

# Part D: One-line diagram

Draw: PV -> controller -> battery -> inverter/DC loads. Show return paths, protection, disconnects, labels, and meter points.

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

# Verification and signoff

| Check | Peer | Instructor |
| --- | --- | --- |
| Loads and hours copied correctly |  |  |
| Every calculation shows units |  |  |
| Assumptions match provided values |  |  |
| Answers are reasonable and rounded clearly |  |  |
| Diagram includes both directions/return path |  |  |
| Protection, disconnects, labels, and test points shown |  |  |