RCS03 | Morning Quiz - Answer Key

CTS Cohort 6 | CTS Day 11 | 2026-07-22 | 10:00 AM-3:00 PM | 6 students

Electrical Fundamentals: Receptacles and Box Layout

1. What conductor normally carries current from the source to the load?

The ungrounded or hot conductor.

2. What conductor normally provides the return path?

The grounded or neutral conductor.

3. What is the purpose of the equipment-grounding conductor?

It provides a low-impedance fault-current path and bonds equipment; it is not a normal return path.

4. What must be proven before continuity mode is used?

The circuit is de-energized and isolated.

5. What is a tug test checking?

That a termination is mechanically secure.

6. Why must copper not extend loosely outside a terminal?

It can contact other parts and create a fault or shock hazard.

7. May this mock-up be connected to building power?

No.

8. Name the four verification stages used today.

De-energized verification, visual check, mechanical/tug check, and continuity/meter check.

9. Who approves the box layout before drilling?

The instructor.

10. Does course practice authorize unlicensed field electrical work?

No.

# Scoring guidance

One point per item. Accept technically equivalent wording. Use errors to reteach before lab release. Safety-boundary errors must be corrected explicitly before the student begins the related task.