# Day 38 EBIKE101 Instructor Run Sheet

Program: CTS Cohort 6

Course: EBIKE101

Week: Final week of CTS / Week 2 of EBIKE101

Date: Tuesday, 2026-09-08

Context: Monday 2026-09-07 was a holiday. Resume with a practical diagnostic day.

Daily focus: Diagnose the E-bike Homunculous, identify what is needed to finish it, and begin safe finish work.

## Big idea

A project is not “almost done” because it looks like a bike. It is close to done when the team can explain what is installed, what is missing, what is verified, what is unsafe, and what the next safe work step is.

## Student-facing goal

By the end of today, students should be able to:

1. Inventory an unfinished e-bike build.

2. Separate mechanical problems from electrical/control problems.

3. Identify missing parts or unknowns.

4. Trace source, protection, control, and load paths.

5. Create a finish-work list that is safe, ordered, and realistic.

6. Begin work only on tasks that are safe and approved.

## Materials

E-bike Homunculous project

Basic hand tools

PPE

Multimeters

Zip ties / wire management materials

Labels / tape / marker

Notebook or printed diagnostic worksheet

Camera or phone for evidence photos

Known-good charger/battery info if available

Any available controller/display/PAS/throttle documentation

## Safety boundary

Students may inspect, document, trace, label, and perform approved mechanical/wire-management work. Battery connection, powered testing, controller testing, and motor testing require instructor control.

Do not allow:

Battery shorting experiments

Continuity testing on live battery circuits

Bypassing fuses

Full-throttle tests

Ride testing before mechanical and electrical checks

Hands near chain, spokes, wheel, or drivetrain during powered testing

## Schedule

### 10:00–10:15 — Arrival / reset / context

Remind students Monday was a holiday and today restarts the build thread.

Name the project: E-bike Homunculous diagnosis and finish work.

Frame the day as “find out what this bike needs,” not “make it work at any cost.”

Instructor line:

“We are not trying to prove the bike works. We are trying to prove what is true about it.”

### 10:15–10:35 — Morning quiz

Give student quiz.

Collect or review immediately.

Use missed questions to decide reteach focus.

### 10:35–10:55 — Safety and system map mini-lesson

Cover:

Source: battery/charger

Protection: fuse/breaker/BMS/controller limits

Control: display, PAS, throttle, brake cutoff, key/switch

Load: motor, lights, accessories

Frame/mechanical: brakes, wheels, torque arms, chain, mounts

Key point:

A silent motor does not prove a bad motor. It can be missing power, missing control input, active brake cutoff, bad connector, wrong display/controller pairing, bad PAS, blown fuse, or mechanical lockup.

### 10:55–11:30 — Whole-bike inventory

Students work in pairs or one full group depending on class size.

Inventory categories:

Frame and mounting

Wheels/tires/brakes

Battery and mount

Controller and mount

Display/control parts

PAS/throttle/brake cutoffs

Motor wiring

Connectors

Protection/fusing

Missing parts

Unknowns

### 11:30–12:00 — Diagnostic board

Create a board with four columns:

1. Verified

2. Unknown

3. Missing/Needed

4. Unsafe until fixed

Students must support every claim with observation, measurement, or documented uncertainty.

### 12:00–12:30 — Lunch / reset

Before break, make the bike safe:

Battery disconnected or controlled by instructor

Tools reset

Loose parts contained

Wires not left under tension

### 12:30–1:00 — Finish-work plan

As a class, choose approved tasks:

Mechanical tightening/inspection

Brake adjustment

Cable routing

Labeling

Connector inspection

Mounting cleanup

Parts list

Documentation/photos

Non-powered continuity checks only where appropriate

Do not jump straight to powered testing because impatience is not an electrical method.

### 1:00–2:20 — Approved work block

Students rotate through roles:

Lead mechanic: physical fitment, fasteners, brakes

Electrical tracer: source/control/load path map

Scribe: worksheet, parts list, evidence log

Safety observer: pinch points, sharp edges, battery boundary

Parts runner: organizes missing/needed components

Instructor circulates and signs off before any higher-risk work.

### 2:20–2:45 — Controlled test decision

Do not guarantee powered testing. Decide from evidence.

Powered test may happen only if:

Wheel is secure

Brakes function

Axle/torque reaction is controlled

Battery voltage/rating is appropriate

Fuse/protection path is present

No damaged/melted wiring

Connectors are seated and strain-relieved

Drive wheel is safely lifted or restrained

Instructor controls first power-up

### 2:45–3:00 — Closeout

Students complete:

What we verified

What is still unknown

Parts needed

Next safe action

Photo evidence list

## Instructor closeout notes

Today is successful if the class leaves with a clear diagnostic state, not necessarily a finished bike. That distinction will save you from the ancient human urge to call chaos “progress” because something briefly spun.