# RCS01 Afternoon Lab - Rolling Sink Module

CTS Cohort 6 | D09 / RCS01 | Monday, July 20, 2026

Core rule: A professional does not guess. A professional checks, protects, and verifies.

# Lab Mission

Build a simple sink drain and P-trap assembly in the rolling lab module, inspect it, leak-test it, troubleshoot problems, and document verified work.

# Safety Boundary

Water stays off until the assembly has been inspected.

No electrical tools or cords near wet areas.

Wipe spills immediately.

Stop if a fitting does not align or a part is missing.

Do not overtighten plastic fittings. Hand-tight plus instructor guidance only.

# Roles

| Role | Job |
| --- | --- |
| Builder | Assembles parts and calls out each step. |
| Inspector | Checks alignment, washers, tightness, and leaks. |
| Documenter | Records parts, issues, fixes, and evidence photos. |

# Parts Inventory

| Part | Have it? | What it does | Notes |
| --- | --- | --- | --- |
| Sink basin/module |  | Holds water and routes it to drain |  |
| Faucet |  | Controls water into sink |  |
| Supply line/shutoff |  | Brings water to faucet and allows shutoff |  |
| Tailpiece |  | Connects sink drain to trap |  |
| P-trap |  | Holds water seal and service point |  |
| Slip nuts/washers |  | Seal adjustable drain joints |  |
| Drain outlet |  | Carries water away |  |

# Build Sequence

Inventory all parts.

Lay parts out in system order.

Dry-fit the tailpiece, trap, and outlet path.

Check washer direction and slip nut alignment.

Hand-tighten fittings.

Stop for inspection before water testing.

Place bucket/towel before testing.

Run a small amount of water and observe.

Mark or record any leak before fixing it.

Correct one issue at a time.

Retest and verify dry joints.

Reset the module after instructor approval.

# Leak Troubleshooting Table

| Problem observed | Likely cause | Fix attempted | Retest result | Instructor initials |
| --- | --- | --- | --- | --- |

# Success Criteria

Parts identified and assembled in correct order.

No leaks after correction and retest.

Group can explain the water path.

Station is clean, dry, and reset.

Verification log is complete.