# PV101 Day 8 Student Print Packet

Compiled packet

Day_08_PV101_Final_Review_and_Assessment_2026-07-16

Contents follow in instructional order.

# PV101 Day 8 Morning Quiz

Student copy | 10-15 minutes

Day_08_PV101_Final_Review_and_Assessment_2026-07-16

Name: __________________________________  Date: __________________

Answer briefly. Show work and units where needed.

1. A 24 V training load draws 2 A. What power does it use?
__________________________________________________________________________________
__________________________________________________________________________________

2. A 40 W device operates for 2 hours. What daily energy does it use?
__________________________________________________________________________________
__________________________________________________________________________________

3. Before testing continuity across a fuse, name the required energy state and meter function.
__________________________________________________________________________________
__________________________________________________________________________________

4. A wire label says polarity is correct. What evidence should verify the label?
__________________________________________________________________________________
__________________________________________________________________________________

5. Why write the expected value and unit before taking a measurement?
__________________________________________________________________________________
__________________________________________________________________________________

# PV101 Day 8 Review Worksheet

Student copy | Use before the final

Day_08_PV101_Final_Review_and_Assessment_2026-07-16

Name: __________________________________  Date: __________________

# Formula and units check

Write the power formula: __________________

Calculate 24 W used for 3 h: __________________ Wh

Convert 0.5 kW to watts: __________________ W

In series, __________________ adds. In parallel, __________________ adds.

# Verification chain

Put in order: evidence, symptom, conclusion, test.
Answer: __________________________________________________________

# System path

Draw and label the basic PV-to-load energy path. Mark one fuse, one disconnect, and one meter point.

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

# Common misconceptions to correct

1. Wire color proves polarity.  Correct / Incorrect: __________
Correction: __________________________________________________________

2. Continuity is tested on an energized circuit.  Correct / Incorrect: __________
Correction: __________________________________________________________

3. Replacing a fuse proves the fault is repaired.  Correct / Incorrect: __________
Correction: __________________________________________________________

4. Loads are the starting point for sizing.  Correct / Incorrect: __________
Correction: __________________________________________________________

5. Cleanup is part of technical work.  Correct / Incorrect: __________
Correction: __________________________________________________________

# PV101 Day 8 Guided-Practice Guide

Student-facing

Day_08_PV101_Final_Review_and_Assessment_2026-07-16

Name: __________________________________  Date: __________________

# Purpose

Provide the student-facing sequence for review, final assessment, reflection, and PV101 closeout.

# Materials

Readiness quiz

Review worksheet

Calculator and pencil

Final assessment

Reflection questions

# Student sequence

Complete the readiness quiz.

Participate in cumulative review.

Use lunch as a test-content break.

Complete the final independently.

Turn in the final before starting reflection.

Submit reflection and leave the room reset.

# Test rules

One point per question.

Show units on calculations.

Ask only for clarification of directions.

Do not discuss or photograph test content.

# Cleanup

Return calculators.

Submit every test and scratch page.

Upload/submit reflection as directed.

Clear tables and wait for dismissal.

# PV101 Day 8 Reflection Questions

Student copy

Day_08_PV101_Final_Review_and_Assessment_2026-07-16

Name: __________________________________  Date: __________________

1. What PV101 skill improved most for you?
__________________________________________________________________________________
__________________________________________________________________________________

2. Which safety or verification rule will you remember?
__________________________________________________________________________________
__________________________________________________________________________________

3. Which topic still needs practice?
__________________________________________________________________________________
__________________________________________________________________________________

4. Describe one time evidence changed your conclusion.
__________________________________________________________________________________
__________________________________________________________________________________

5. What does professional cleanup/documentation mean to you?
__________________________________________________________________________________
__________________________________________________________________________________

# PV101 Final Assessment

Student copy | 25 questions | 25 points

Day_08_PV101_Final_Review_and_Assessment_2026-07-16

Name: __________________________________  Date: __________________  Score: ______ / 25

Directions: Work independently. Circle one answer for multiple choice. Show formulas and units for calculations. Write clearly for explanations. Each question is worth one point.

## 1. Photovoltaic (PV) equipment converts which resource directly into electricity?

A. Moving air

B. Light

C. Flowing water

D. Fuel vapor

## 2. Which application most clearly needs energy storage to operate when sunlight is unavailable?

A. A direct daytime PV fan

B. A grid-tied array with no backup requirement

C. An off-grid cabin used at night

D. A module demonstration with no load

## 3. Before changing wiring on a training PV system, the safest first action is to:

A. Work faster

B. Isolate energy sources and verify the state

C. Trust the switch label

D. Replace the fuse

## 4. A student is uncertain about polarity. What should happen?

A. Connect briefly and watch

B. Ask a classmate to guess

C. Stop and verify with the correct meter setup

D. Reverse both conductors

## 5. Which statement is correct?

A. Voltage is electrical pressure and current is flow

B. Current is measured in volts

C. Power is measured in ohms

D. Resistance creates energy

## 6. A 12 V fan draws 3 A. Calculate its power in watts. Show the formula.

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

## 7. Peak sun hours are used mainly to estimate:

A. Daily equivalent full-sun energy

B. Wire color

C. Battery terminal size

D. AC frequency

## 8. What usually happens to PV output when a module is heavily shaded?

A. Output improves

B. Output is reduced

C. AC voltage doubles

D. Battery capacity increases

## 9. A PV module exposed to light should be treated as:

A. Always de-energized

B. A possible DC voltage source

C. An AC-only device

D. A fuse

## 10. Read this connection diagram: [Module 1] -- series -- [Module 2]. Which electrical quantity normally adds in this matched series connection?

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

## 11. What is the main job of a charge controller in a battery-based PV system?

A. Convert DC to AC

B. Regulate charging to protect the battery

C. Create sunlight

D. Replace overcurrent protection

## 12. What is the main job of an inverter?

A. Convert DC to AC

B. Store energy

C. Measure resistance

D. Increase sun hours

## 13. Three 10 W lights operate 4 hours per day. Calculate their total daily energy use.

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

## 14. What should be completed before selecting battery, array, or inverter size?

A. A load assessment

B. A wire-color chart

C. A tool inventory only

D. A final photograph

## 15. Why is a fuse placed close to a battery source?

A. To increase voltage

B. To protect as much conductor as practical

C. To make the inverter quieter

D. To eliminate shutdown steps

## 16. A long undersized conductor carrying current is most likely to cause:

A. Lower voltage drop

B. Voltage drop and heating

C. More sun hours

D. Increased battery capacity

## 17. Which is a reasonable advantage of a ground-mounted array?

A. It always costs less

B. It can be easier to access and orient

C. It cannot be shaded

D. It needs no structure

## 18. A roof area is shaded from 1:00-4:00 PM, while an open ground area has clear solar access. Circle the better conceptual location and write one reason.

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

## 19. After observing a fault symptom, the best next step is to:

A. Replace several parts

B. Choose one test that can separate likely causes

C. Bypass protection

D. Trust the label

## 20. Explain why replacing a blown fuse is not enough to prove the system is repaired.

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

## 21. In a permitting context, the AHJ is the organization or official that:

A. Guarantees system output

B. Sets the price of every module

C. Enforces the applicable local requirements and approvals

D. Replaces the serving utility

## 22. A current, documented rebate in a training example covers $500 of a $2,500 cost. Calculate the remaining cost before any other fees or credits.

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

## 23. After completing a set of practice questions, explain why reviewing the missed questions is more useful than recording only the score.

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

## 24. A cumulative practice question asks you to convert 0.75 kW to watts. Show the conversion.

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

## 25. A small PV training system does not power its load. The panel is in light, the controller display is off, and a label says the battery fuse is good. Describe the safe first three actions/tests and the evidence you would document.

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________