INSTRUCTOR PACKET

EBIKE101 Day 3

Motors, Controllers, Throttle/PAS, and Signal Flow

Evidence first. Motion last.

# Morning-of launch

Print one student packet per student.

Display the matching 20-slide deck.

Stage one visible e-bike or conversion bike with battery off or disconnected.

Put SIGNAL PATH and POWER PATH on the board in two colors.

Stage loose components, removable labels, basic tools, and a multimeter for instructor use.

# Learning targets

Separate signal and power paths.

Describe motor and controller jobs.

Explain throttle, PAS, display, and brake-cutoff inputs.

Trace battery to controller to motor.

Build an ordered no-assist diagnostic plan.

## Core rule

Power off first. Trace before testing. Control motion before adding power.

# Five-hour run of show

| Time | Block | Instructor move |
| --- | --- | --- |
| 10:00-10:10 | Landing | Attendance and Day 1 safety review |
| 10:10-10:30 | Quiz | Check safe thinking; reteach before hands-on work |
| 10:30-10:55 | Two paths | Separate commands from working energy |
| 10:55-11:20 | Motor | Torque, hub-drive, and mid-drive |
| 11:20-11:45 | Controller | Inputs, decision, and controlled output |
| 11:45-12:00 | Safety inputs | Throttle/PAS requests; brake cutoff overrides |
| 12:00-12:30 | Lunch | Bike stable and power off |
| 12:30-1:05 | Lab A | Identify and tag components |
| 1:05-1:45 | Lab B | Draw and compare both paths |
| 1:45-2:00 | Break | Reset labels and tools |
| 2:00-2:35 | Lab C | Trace one no-assist scenario |
| 2:35-2:55 | Debrief | Defend first safe test with evidence |
| 2:55-3:00 | Close | Preview battery and charging safety |

# Teaching sequence

## 1. Power path

Battery -> protection -> controller -> motor -> mechanical rotation

## 2. Signal path

Throttle, PAS, brakes, and display -> controller decision

## 3. Controller

The controller is where the paths meet. It reads requests and safety conditions, then controls energy sent to the motor.

## 4. Diagnostic order

Make safe

Confirm the complaint

Trace the path

Test one checkpoint at a time

Record the result

# Stop-work triggers

Unexpected motor or wheel motion

Unstable bike or driven wheel

Battery heat, swelling, odor, damage, wetness, or unknown state

Forced, damaged, or mismatched connector

Exposed conductor or uncertain polarity

Wiring near moving parts, sharp edges, heat, or pinch points

Attempt to bypass a cutoff or test by random connection

# Lab setup and fault cards

Use an actual known-safe condition or a paper fault card. Do not create an energized fault. Good scenarios:

Battery switch is off or system wake-up step is missing.

Display mode or assist level prevents assist.

Brake cutoff is active in the scenario.

A low-voltage signal connector is visually identified as unseated while power remains off.

PAS alignment or sensor gap needs inspection.

Motor cable routing or connector condition requires instructor review.

## Backup

Use labeled component cards if no complete bike is available. Teams build both paths, receive one fault card, and defend their checkpoint sequence without applying power.

# Morning Quiz — Answer Key

Use with the student quiz. Correct answers and teaching rationales are listed below.

| # | Correct answer | Why |
| --- | --- | --- |
| 1 | B. Make the system safe and power it off | Make the system safe before tracing it. |
| 2 | C. Read inputs and manage power sent to the motor | The controller reads rider and safety inputs, then controls motor power. |
| 3 | A. Converts electrical energy into mechanical rotation | The motor turns controlled electrical energy into mechanical rotation. |
| 4 | A. A direct request for motor assist | A throttle sends a direct assist request to the controller. |
| 5 | C. Pedaling input | PAS detects pedaling activity or cadence input. |
| 6 | B. Tell the controller to stop motor drive | A safety input should override a go input. |
| 7 | A. Battery -> protection -> controller -> motor | The high-energy path runs from battery through protection and controller to motor. |
| 8 | C. Throttle, PAS, brakes, and display | These devices provide rider, mode, or safety signals. |
| 9 | B. Signal carries a command; power carries energy | A command and the energy that performs it are different paths. |
| 10 | C. Make safe and confirm the complaint | Confirm the condition safely before changing the system. |
| 11 | B. Identify and verify it before connection | Unknown connectors must be identified, not forced or guessed. |
| 12 | A. Clear of moving parts, sharp edges, heat, and pinch points | Safe routing prevents abrasion, snagging, heat damage, and shorts. |
| 13 | C. Stable with the driven wheel and area controlled | Stability and area control prevent unexpected motion from becoming injury. |
| 14 | B. At the wheel hub | A hub motor is built into or drives the wheel hub. |
| 15 | A. Through the bicycle drivetrain | A mid-drive adds torque through the chain and bicycle drivetrain. |
| 16 | C. A recorded checkpoint and result | Documentation creates repeatable evidence. |
| 17 | B. Stop-work and make the system safe | Unexpected motion is a stop-work trigger. |
| 18 | A. Shape, keying, pin count, labels, and routing | Multiple identifiers are stronger evidence than color alone. |
| 19 | C. Instructor approval, stable bike, clear area, and controlled motion | A powered test requires control, approval, and a clear area. |
| 20 | B. Trace the system instead of guessing at parts | Ordered tracing isolates the problem without random part swapping. |

# Lab scoring

| Category | 3 - Job-ready | 2 - Acceptable | 1 - Needs rework | 0 - Unsafe |
| --- | --- | --- | --- | --- |
| Safety | Holds power-off boundary | Minor reminder | Repeated reminders | Unsafe action |
| Identification | All core parts correct | One minor miss | Several errors | Cannot identify path |
| Diagram | Both paths clear | Mostly correct | Paths mixed | No usable diagram |
| Diagnosis | Ordered evidence plan | Usable basic plan | Guessing remains | Random connection or bypass |
| Documentation | Clear checkpoints | Basic notes | Incomplete notes | No record |

# Successful completion

Students can name the path, mark the checkpoints, explain the first safe test, and stop when evidence does not support the next step.

## Finish early

Give teams a second paper fault card and require them to identify which checkpoint changes and which checkpoints stay the same.