| CTS Cohort 6 ¦ PV101 Day 07 | Battery Storage, Solar Needs, and Wire Sizing ¦ 2026-07-15 |
| --- | --- |

PV101 Day 7 - Battery Storage vs. Solar Needs and Wire Sizing

Student Worksheet - print ready

| Name: ____________________ | Team: __________ | Date: __________ | Instructor: __________ |
| --- | --- | --- | --- |

Today's big idea A solar system has two different needs: making energy and storing energy. A panel is the faucet. A battery is the tank. Wire is the road. Fuses are the guardrails. If one part is undersized, the whole system becomes unreliable.

Core formulas for today Watts = Volts x Amps Watt-hours per day = Watts x Hours used per day Battery watt-hours = Battery volts x Amp-hours Usable storage = Battery watt-hours x usable percent Solar per day = Panel watts x peak sun hours x system efficiency Amps = Watts / Volts Fuse rule: protect the wire first. Fuse size must not exceed what the wire can safely carry.

# Part 1 - Load list: How much energy do we need?

Directions: Use Watts x Hours = Watt-hours. Fill in the last column. Round to the nearest whole watt-hour.

| Load / device | Volts | Amps or Watts given | Hours per day | Wh/day |
| --- | --- | --- | --- | --- |
| LED lights | 12 V | 2 A | 4 hr | __________ |
| Roof vent fan | 12 V | 3 A | 3 hr | __________ |
| Phone charging station | 12 V | 24 W | 2 hr | __________ |
| Water pump | 12 V | 5 A | 0.5 hr | __________ |
| Small inverter load | 120 V AC | 150 W | 1 hr | __________ |
| Your added load: __________ | ____ | ____ | ____ | __________ |

Total daily energy need: __________________ Wh/day

One sentence summary: This system needs about __________________ Wh each day because ________________________________________________.

# Part 2 - Battery storage: How big is the tank?

Given battery for this exercise 12.8 V LiFePO4 battery, 50 Ah. For class design work, use 80% usable storage unless the instructor says otherwise.

| 1. Battery watt-hours = 12.8 V x 50 Ah | __________________ Wh |
| --- | --- |
| 2. Usable storage = battery Wh x 0.80 | __________________ Wh usable |
| 3. If your daily load is the Part 1 total, will one battery cover one day? | Yes / No because ________________________________ |
| 4. How many of these batteries would give roughly two days of usable storage? | __________________ batteries |
| 5. Why is it not smart to drain a battery to empty every day? | ________________________________________________ |

Quick check: More battery storage helps at night and on cloudy days. More panel helps recharge during daylight. They solve different problems.

# Part 3 - Solar needs: How fast can we refill the tank?

Class assumption Peak sun hours are not the same as sunrise-to-sunset hours. Today, use 4 peak sun hours and 75% system efficiency. That 75% includes real-world losses from heat, wiring, controller behavior, angle, and imperfect conditions.

Formula: Solar Wh/day = Panel watts x peak sun hours x efficiency

| Panel setup | Formula | Solar Wh/day | Enough for your Part 1 load? |
| --- | --- | --- | --- |
| One 100 W panel | 100 W x 4 hr x 0.75 | __________ | Yes / No |
| One 200 W panel | 200 W x 4 hr x 0.75 | __________ | Yes / No |
| Two 200 W panels | 400 W x 4 hr x 0.75 | __________ | Yes / No |
| Your design: ______ W | ____ W x 4 hr x 0.75 | __________ | Yes / No |

Explain the mismatch if there is one: __________________________________________________________________________________________

If the solar makes less energy than the loads use each day, what happens over several days? __________________________________________

# Part 4 - Wire sizing: How big does the road need to be?

Important safety note This is a training worksheet, not a final code design. Real installations require manufacturer instructions, fuse ratings, temperature ratings, wire type, length, voltage drop, and applicable electrical code. Today we are practicing the thinking process.

Step 1: Find current. Use Amps = Watts / Volts when watts are given.

| Circuit | Voltage | Load | Current | First wire-size thought |
| --- | --- | --- | --- | --- |
| LED lights | 12 V | 2 A | 2 A | Small load, still fuse it |
| Vent fan | 12 V | 3 A | 3 A | Small load, length matters |
| Water pump | 12 V | 5 A | 5 A | Higher startup surge |
| 200 W panel into controller | About 12 V system | 200 W | ________ A | Controller/wire must handle current |
| 300 W inverter on 12 V battery | 12 V | 300 W | ________ A | Much higher DC current than people expect |
| 600 W inverter on 12 V battery | 12 V | 600 W | ________ A | Large DC current, short thick wire |

What surprised you about inverter current on the 12 V side? ________________________________________________________________

# Part 5 - Simplified wire and fuse thinking

Rule of thumb for this class Wire must be sized for current and distance. Fuse protects the wire. A fuse that is too large can allow the wire to overheat before the fuse opens. A fuse that is too small may nuisance-blow during normal use.

| Training current range | Training wire-size thought | Fuse thinking |
| --- | --- | --- |
| 0-5 A | Small branch circuit; keep runs neat and protected | Fuse above normal load, below wire limit |
| 5-15 A | Medium branch circuit; check length and voltage drop | Common small PV/load range |
| 15-30 A | Larger DC circuit; wire size matters fast | Controller/inverter input territory |
| 30 A+ | High current; short runs, thick wire, serious protection | Do not guess |
| Unknown | Stop and calculate or look up rating | No mystery fuses |

# Part 6 - Team design challenge

Scenario: You are planning a small camper/RV learning system. It should run the listed loads for one normal day and recover with solar during decent sun. Use the assumptions from this worksheet.

| 1. Total daily load from Part 1 | __________________ Wh/day |
| --- | --- |
| 2. Battery usable storage per 12.8 V 50 Ah battery | __________________ Wh usable |
| 3. Number of batteries you recommend | __________________ |
| 4. Panel watts you recommend | __________________ W |
| 5. Solar Wh/day using 4 sun hours and 75% efficiency | __________________ Wh/day |
| 6. Highest-current circuit in your design | __________________ |
| 7. First circuit you would fuse and why | ________________________________________________ |
| 8. One thing you would verify with a meter before turning it on | ________________________________________________ |

# Part 7 - Reflection and exit ticket

1. In your own words, what is the difference between solar need and battery/storage need?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

2. Why can a small 12 V inverter pull a surprisingly large amount of current from a battery?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

3. What is one mistake a beginner might make when choosing wire or fuse size?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

4. Today's professional habit: Before replacing parts, I will ________________________________________________.

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

Exit phrase Panels make energy. Batteries store energy. Wires carry current. Fuses protect wires. Professionals verify before they build.