RCS02 | Student Packet

CTS Cohort 6 | CTS Day 10 | 2026-07-21 | 10:00 AM-3:00 PM | 6 students

Residential Framing Basics: Measure, Layout, Build

Core rule: A professional does not guess. A professional checks, protects, and verifies.

Big idea: Accurate framing begins before the saw starts: read the plan, measure twice, mark clearly, and verify the cut list.

# Learning targets

Read a tape measure to 1/16 inch and add common fractions.

Identify plates, studs, king studs, jack studs, header, sill, and cripples.

Lay out a 4-foot by 3-foot training wall with a framed window opening.

Complete an instructor-approved cut list and assemble a square wall frame.

# Today's run

10:00-10:15 | Arrival, PPE, retrieval warm-up

10:15-10:30 | Morning quiz

10:30-10:50 | Quiz review and measurement misconception check

10:50-11:20 | Tape-measure and fraction model

11:20-12:00 | Wall-member map and window-framing demonstration

12:00-12:30 | Cut-list guided practice and lab release

12:30-1:00 | Lunch and controlled reset

1:00-2:25 | Individual capstone framing lab

2:25-2:45 | Dimensions, diagonal QC, evidence, cleanup

2:45-3:00 | Exit ticket and Day 3 preview

# Key words

top plate | bottom plate | stud | king stud | jack stud | header | sill | cripple | on-center | crown | square

# Guided worksheet

1. Convert: 1/2 = __/16; 3/4 = __/16; 7/8 = __/16.

________________________________________________________________________________

________________________________________________________________________________

2. Add: 14 3/8 + 9 7/16. Show the denominator step.

________________________________________________________________________________

________________________________________________________________________________

3. Label the wall members on the instructor-provided drawing.

________________________________________________________________________________

________________________________________________________________________________

4. Write the cut-list quantity, length, and purpose for each member.

________________________________________________________________________________

________________________________________________________________________________

5. Record two diagonal measurements and explain what they show about square.

________________________________________________________________________________

________________________________________________________________________________

# Afternoon lab mission

Each student begins an individual 4-foot by 3-foot wall capstone with a framed window opening. Students inventory stock, complete the approved cut list, mark and verify all members, make supervised cuts only after clearance, dry-fit, fasten, and complete a square/dimension inspection.

## Safety boundary

Eye and hearing protection in active cutting zones

Instructor authorization before each cutting operation

Guards stay installed and functional

Clamp or support stock

Wall stays flat until instructor authorizes movement

Stop work for conflicting measurements or damaged material

## Build / test sequence

[ ] PPE and station inspection

[ ] Read training drawing

[ ] Create cut list

[ ] Mark and partner-verify

[ ] Instructor cut authorization

[ ] Supervised cutting

[ ] Dry layout and member labels

[ ] Fasten frame

[ ] Measure diagonals and correct

[ ] Photograph, document, reset

# Evidence record

[ ] Completed measurement sheet: ______________________________________________

[ ] Approved cut list: ______________________________________________

[ ] Labeled dry layout photo: ______________________________________________

[ ] Final wall dimensions and diagonal check: ______________________________________________

[ ] Instructor tool-control observation: ______________________________________________

# Reflection and exit ticket

One measurement skill I can now demonstrate

________________________________________________________________________________

________________________________________________________________________________

One wall member and its job

________________________________________________________________________________

________________________________________________________________________________

One verification I completed

________________________________________________________________________________

________________________________________________________________________________

One step I need to practice

________________________________________________________________________________

________________________________________________________________________________

# Instructor verification

Status: [ ] Approved  [ ] Correct and recheck  [ ] Needs follow-up

Instructor initials: __________________  Date/time: __________________