# Day 38 EBIKE101 Student Print Packet

CTS Cohort 6 | EBIKE101 | Tuesday 2026-09-08

# Day 38 EBIKE101 Morning Quiz — Student Copy

**Program:** CTS Cohort 6

**Course:** EBIKE101

**Date:** Tuesday, 2026-09-08

**Topic:** E-bike Homunculous diagnosis and finish-work planning

**Name:** _______________________________

## Directions

Circle the best answer for each question. This quiz is about safe diagnosis, not guessing bravely at wires like a tiny electrician raccoon.

---

## 1. Before diagnosing an e-bike electrical problem, what should happen first?

A. Connect every loose wire and see what turns on

B. Confirm the system is safe to inspect and de-energized when appropriate

C. Spin the motor by hand as fast as possible

D. Replace the controller immediately

## 2. What is the best first step when looking at an unfinished e-bike build?

A. Make a full system inventory and note what is present, missing, loose, or unknown

B. Assume the last person wired it correctly

C. Start cutting zip ties so every wire is free

D. Connect the battery and test under load

## 3. What is the job of a fuse or breaker in an e-bike power circuit?

A. Make the motor run faster

B. Stop unsafe current before wiring or parts are damaged

C. Increase battery voltage

D. Turn pedal assist on automatically

## 4. If the display does not power on, which item is a reasonable early check?

A. Tire tread pattern

B. Battery voltage, main switch, fuse, and display/controller connector

C. Seat height only

D. Rear reflector angle

## 5. What does a missing throttle usually mean during diagnosis?

A. The whole project is impossible

B. The team should document the missing control input and verify whether the system can operate by PAS/display without it

C. The battery must be bad

D. The motor wires should be reversed

## 6. Why should wire color not be trusted by itself?

A. Wire color is always random

B. Wire color is a clue, but repairs, adapters, and different manufacturers can change meanings

C. Only red wires carry electricity

D. Black wires are never connected to anything

## 7. What is the safest way to use a continuity test?

A. On a de-energized circuit or component, following meter instructions

B. Across a live battery pack

C. Across the controller’s main power leads while the battery is connected

D. Anywhere, because continuity mode is harmless

## 8. What can happen if an e-bike brake cutoff sensor is active or misadjusted?

A. The controller may prevent motor output

B. The battery voltage will double

C. The tire pressure will drop

D. The chain will automatically tighten

## 9. What does PAS stand for in most e-bike systems?

A. Pedal Assist Sensor/System

B. Power Amplifier Switch

C. Positive Axle Support

D. Panel Alignment Screw

## 10. Why does motor cable strain relief matter?

A. It makes the bike look expensive

B. It prevents cable damage from pulling, rubbing, bending, or wheel movement

C. It increases top speed

D. It changes the tire size

## 11. If a fuse is blown, what should the team do before simply replacing it?

A. Bypass it with wire

B. Ask what caused the fuse to blow and inspect for shorts, wrong connections, or overload

C. Install the largest fuse available

D. Tape over the fuse holder

## 12. What does it mean to trace a circuit?

A. Draw a pretty version of the bike after it works

B. Follow the actual path from source, through protection/control, to the load

C. Guess which wire is correct based on color

D. Skip testing because the labels look professional

## 13. Which mechanical item is critical before powered wheel testing?

A. Handlebar grip color

B. Axle nuts/torque arms, wheel security, brake function, and clear chain/wheel movement

C. Sticker placement

D. Bell volume

## 14. What is a good practice for first powered motor testing in class?

A. Full throttle with someone holding the rear wheel

B. Secure the bike, keep people clear of moving parts, use low power, and have instructor control

C. Ride it through the shop immediately

D. Test with no brakes installed

## 15. What should be recorded in a diagnostic work log?

A. Only the final answer

B. Symptoms, observations, measurements, changes made, parts needed, and next step

C. Only who touched the bike last

D. Nothing if the team feels confident

## 16. What does a controller usually do in an e-bike system?

A. Converts rider/display inputs into controlled power sent to the motor

B. Inflates the tires

C. Replaces the chain

D. Makes the battery physically larger

## 17. What should happen if a component smells burnt, gets unusually hot, or shows melted wiring?

A. Keep testing to gather more data

B. Stop, disconnect power if safe, and get instructor review

C. Spray it with water

D. Wrap it in tape and continue

## 18. Why is connector inspection important?

A. Small pins, bent terminals, loose plugs, corrosion, or mismatched connectors can stop a system from working

B. Connectors are only cosmetic

C. Connectors never fail

D. Every connector can plug into every other connector safely

## 19. What is the difference between a symptom and a root cause?

A. They are always the same thing

B. A symptom is what you observe; a root cause is the underlying reason it is happening

C. A root cause is always the most expensive part

D. A symptom only matters if the motor spins

## 20. When is the Homunculous closer to “finished”?

A. When one person says “looks good”

B. When mechanical safety, electrical safety, control function, wiring support, needed parts, and documentation are verified

C. When every wire is hidden

D. When the fastest student rides it first

---

## Exit check

Write one thing your team must verify before this e-bike should be powered or ridden:

______________________________________________________________________________

______________________________________________________________________________

# Day 38 EBIKE101 Diagnostic Worksheet

**Program:** CTS Cohort 6

**Course:** EBIKE101

**Date:** Tuesday, 2026-09-08

**Project:** E-bike Homunculous

**Team members:** ___________________________________________

## Purpose

Use this worksheet to diagnose what the Homunculous needs before it can be safely finished, powered, and eventually ridden.

## Safety boundary

Do not connect the battery, bypass protection, open battery cases, test live battery circuits in continuity mode, spin the motor under power, or ride the bike without instructor approval.

## 1. Whole-bike inventory

## 2. System trace

Fill this out before powered testing.

## 3. Symptom list

## 4. Missing parts / supplies list

## 5. Safe work approved today

Only list work the instructor has approved.

## 6. Evidence log

## 7. Exit reflection

What is one thing we verified today?

______________________________________________________________________________

What is one thing we still do not know?

______________________________________________________________________________

What is the next safest work step?

______________________________________________________________________________

# Day 38 EBIKE101 Afternoon Lab — Homunculous Finish-Work Diagnosis

**Program:** CTS Cohort 6

**Course:** EBIKE101

**Date:** Tuesday, 2026-09-08

**Lab type:** Live project diagnosis and approved finish work

## Lab objective

Students will diagnose the current state of the E-bike Homunculous, identify what is required to finish it, and begin only the safe work that has been verified and approved.

## Instructor setup

Before students begin:

1. Place the bike where all sides can be accessed.

2. Keep battery connection under instructor control.

3. Remove or isolate any obviously unsafe condition.

4. Stage hand tools, meter, labels, zip ties, and worksheet.

5. Define the boundary: diagnosis first, power later only if earned by evidence.

## Lab roles

Assign or rotate:

Mechanical lead: checks frame, wheels, brakes, fasteners, chain path.

Electrical tracer: maps source, protection, controller, controls, motor.

Documentation lead: worksheet, board, photo evidence, parts list.

Safety observer: watches pinch points, battery boundary, tool use, moving parts.

Parts lead: organizes missing parts, hardware, adapters, labels.

## Part 1 — Mechanical readiness inspection

Students inspect:

Wheel security

Axle nut condition

Torque arm or anti-rotation support if applicable

Brake function

Tire condition and pressure

Chain alignment

Crank/pedal condition

Sharp edges

Cable rub points

Loose fasteners

Battery/controller mounting

Stop if:

Motor wheel can shift in dropout

Brakes cannot stop the wheel

Chain path is unsafe

Battery mount is loose

Any wire can reach tire, chain, brake rotor, or crank

## Part 2 — Electrical/control inventory

Students identify and document:

Battery voltage/rating if labeled

Controller label/rating if visible

Main power connector

Fuse/protection location

Display connector

PAS connector/sensor/magnet ring

Throttle presence or absence

Brake cutoff connectors

Motor cable

Accessory wires

Damaged connectors

Unknown wires

Rule:

Unknown wires stay unknown until traced, labeled, or verified. Guessing does not become truth just because someone says it confidently in a shop voice.

## Part 3 — Diagnostic decision tree

Use this order:

1. Is the bike mechanically safe to inspect?

2. Is the battery appropriate and controlled?

3. Is there protection between battery and controller?

4. Does the controller have a secure mount?

5. Are control inputs present?

6. Are brake cutoffs present and not stuck active?

7. Is the PAS sensor installed and aligned?

8. Is the throttle present, missing, or optional for this controller?

9. Is the motor cable undamaged and supported?

10. What must be fixed before first power-up?

## Part 4 — Approved finish work

Allowed student work may include:

Brake adjustment

Tightening non-critical hardware under supervision

Labeling wires

Improving cable routing

Adding strain relief

Cleaning mounting surfaces

Organizing connectors

Creating a parts-needed list

Documenting evidence

Non-powered meter checks where appropriate

Instructor-only or instructor-controlled work:

Battery connection

First power-up

Controller testing

Motor spin testing

Fuse replacement if cause is unknown

Any modification to high-current wiring

Any ride test

## Part 5 — First power-up gate

Do not perform a powered test unless all are checked:

Mechanical assembly secure

Brakes functional

Drive wheel controlled

Battery voltage/rating appropriate

Fuse/protection path verified

Controller mounted

No melted/damaged wires

No unrestrained motor cables

No hands near moving parts

Instructor gives explicit approval

## Completion target

The lab is complete when the team can answer:

1. What works or appears complete?

2. What is missing?

3. What is unsafe?

4. What is unknown?

5. What should be done next?

Finishing the bike is excellent. Finishing the diagnosis is required.