Student Print Packet

EBIKE101 | CTS Cohort 6 | Day 33 | Monday, August 31, 2026

EBIKE101 | CTS Cohort 6 | Day 33

# EBIKE101 Day 1 Student Packet

Safe micro-mobility starts before the throttle moves.

Name: ______________________________________    Team: ______________________

## Today's big question

What makes an e-bike different from a bicycle, and what must be proven safe before anybody rides it?

## Part 1: Bicycle baseline

Label or describe each part. Use plain language.

| Part | What does it do? | What could make it unsafe? |
| --- | --- | --- |
| Frame |  |  |
| Fork |  |  |
| Front wheel |  |  |
| Rear wheel |  |  |
| Tires |  |  |
| Brakes |  |  |
| Chain/drivetrain |  |  |
| Handlebars/stem |  |  |
| Seat/seatpost |  |  |
| Pedals/cranks |  |  |

## Part 2: E-bike added systems

| E-bike part | Job | What could fail? |
| --- | --- | --- |
| Battery |  |  |
| Battery mount |  |  |
| Motor |  |  |
| Controller |  |  |
| Display |  |  |
| Throttle |  |  |
| Pedal assist sensor |  |  |
| Wiring harness |  |  |
| Connectors |  |  |
| Charger |  |  |
| Brake cutoffs, if installed |  |  |

## Part 3: Basic system flow

Fill in the flow.

Rider input -> ____________________ -> battery power -> ____________________ -> wheel motion

What does the rider control?

______________________________________________________________________________________

What does the controller control?

______________________________________________________________________________________

What does the motor move?

______________________________________________________________________________________

## Part 4: Pre-ride inspection checklist

Mark each item.

P = Pass

F = Fail

R = Needs instructor review

N/A = Not installed or not available today

### Mechanical bike

| Item | P/F/R/N/A | Notes |
| --- | --- | --- |
| Frame has no obvious cracks, damage, or loose parts |  |  |
| Fork looks straight and secure |  |  |
| Wheels are secure |  |  |
| Tires have usable pressure and no obvious damage |  |  |
| Brakes squeeze firmly and stop the bike during a roll check |  |  |
| Handlebars and stem are tight |  |  |
| Seatpost is secure |  |  |
| Chain/drivetrain is present, routed, and not jammed |  |  |
| Pedals/cranks are secure |  |  |

### Electrical assist system

| Item | P/F/R/N/A | Notes |
| --- | --- | --- |
| Battery case has no swelling, heat, odor, or visible damage |  |  |
| Battery is mounted securely |  |  |
| Controller is mounted securely |  |  |
| Motor cable is protected and not rubbing |  |  |
| Connectors are seated and not damaged |  |  |
| Wiring is clear of tires, chain, brakes, cranks, and pinch points |  |  |
| Display/control unit is secure |  |  |
| Throttle/PAS is not being touched during setup |  |  |
| Charger is known, matched, and undamaged |  |  |

### Rider and area

| Item | P/F/R/N/A | Notes |
| --- | --- | --- |
| Helmet available and fits |  |  |
| Controlled area can be marked before any ride test |  |  |
| No public traffic or uncontrolled pedestrian path |  |  |
| Instructor has approved the next test step |  |  |

## Part 5: Team risk report

Write one risk in each category.

Mechanical risk:

______________________________________________________________________________________

Electrical risk:

______________________________________________________________________________________

Rider-control risk:

______________________________________________________________________________________

Area/environment risk:

______________________________________________________________________________________

## Exit ticket

1. What makes an e-bike different from a normal bicycle?

______________________________________________________________________________________

2. What are three things that must be checked before any powered test?

______________________________________________________________________________________

3. What is one thing you will check first tomorrow?

______________________________________________________________________________________