# PV102 Day 5 Student Packet

Battery Storage Autonomy and Critical Loads

PV102 | CTS Day 21 | Student Print Packet
Storage has a defined job: carry named loads through named time. | Monday, August 10, 2026

# Today's rule

Storage has a defined job: carry named loads through named time.

# Learning targets

Explain the difference between battery capacity, usable capacity, and autonomy.

Estimate storage need from essential daily Wh and autonomy goal.

Identify why battery chemistry, BMS limits, temperature, ventilation, and overcurrent protection matter.

Defend load shedding as a design tool, not a failure.

# Morning quiz

1. Battery capacity is commonly expressed as:

A. Ah or Wh

B. Degrees azimuth only

C. Wire color

D. Roof pitch only

2. A battery bank should be sized from:

A. Defined loads and runtime

B. The prettiest battery case

C. Random parts available

D. Module color

3. Usable capacity is often less than nameplate capacity because:

A. Depth-of-discharge and protection limits matter

B. Labels are decoration

C. All loads are AC

D. Sunlight is heavy

4. Autonomy means:

A. Time the system must carry loads without enough solar input

B. A compass direction

C. A breaker type

D. A module frame dimension

5. Load shedding means:

A. Reducing or turning off lower-priority loads

B. Cutting random wires

C. Ignoring the client

D. Making voltage disappear

6. Estimate: 1,000 Wh/day for 2 days = ___ Wh before losses and usable-capacity adjustment.

________________________________________________________________________________________________

________________________________________________________________________________________________

7. Why does battery voltage matter in system design?

________________________________________________________________________________________________

________________________________________________________________________________________________

8. Name one battery safety boundary.

________________________________________________________________________________________________

________________________________________________________________________________________________

9. Why is “more battery” not always the best answer?

________________________________________________________________________________________________

________________________________________________________________________________________________

10. Write the Day 5 rule in your own words.

________________________________________________________________________________________________

________________________________________________________________________________________________

# Reference: decision order

| Step | Action | Proof question |
| --- | --- | --- |
| 1 | Define the job | What must the system accomplish? |
| 2 | Collect evidence | What is measured, observed, or still unknown? |
| 3 | Choose boundary | What must be protected, isolated, sized, or verified? |
| 4 | Defend the choice | What evidence supports the decision? |
| 5 | Handoff | What should the next person know? |

# Critical Load Storage Plan

Teams reduce an oversized wish list into a critical-load storage plan that a real system could support.

## Scenario options

Bus conversion overnight loads: fridge, fan, lights, phone, laptop, water pump, optional induction cooktop.

Remote sensor must survive two stormy days with no site visit.

Small shop wants outage backup for lights, router, one outlet, and refrigerator only.

## Team procedure

Start with the Day 3 load table or provided scenario.

Mark each load as critical, useful, or optional.

Calculate critical daily Wh.

Choose an autonomy target and estimate required Wh before adjustment.

List the battery information required before final sizing.

Handoff the critical-load list, autonomy target, and rejected optional loads.

## Team worksheet

| Prompt | Team response |
| --- | --- |
| System/job statement |  |
| Evidence used |  |
| Decision made |  |
| Safety boundary |  |
| Top unknowns |  |
| Next action |  |

# Exit ticket

Which load did your team remove or limit, and why did that make the system stronger?

________________________________________________________________________________________________

________________________________________________________________________________________________