# PV102 Day 1 Student Packet

Applications and design decisions

| Quiz | Reference | Design lab |
| --- | --- | --- |

Student Print Packet | Audience: Student | Monday, August 3, 2026

Today's rule Start with load, site, and need. Record unknowns instead of guessing.

# Learning targets

Distinguish four common PV architectures.

Map a first-pass energy path.

Use client and site evidence to defend a design choice.

State protection and verification boundaries before construction.

# Morning quiz

1. What type of electrical output is produced directly by a PV module?

A. AC

B. DC

C. Mechanical

D. Thermal

2. When matching PV modules are connected in series, which quantity primarily adds?

A. Voltage

B. Current

C. Sunlight

D. Frequency

3. When matching PV strings are connected in parallel, which quantity primarily adds?

A. Voltage

B. Current

C. Resistance only

D. Frequency

4. For the small battery-based training systems used in PV101, which connection should be made first when the controller manufacturer's procedure requires battery-first startup?

A. PV to controller

B. Load to inverter

C. Battery to controller

D. Ground rod to roof

5. What is the main job of an overcurrent protective device such as a fuse?

A. Increase voltage

B. Limit damaging current

C. Create sunlight

D. Store energy

6. Before using continuity mode, the circuit must be:

A. Energized

B. De-energized and verified

C. Connected to the grid

D. Under full load

7. A 12 V load draws 2 A. What is its power in watts?

________________________________________________________________________________________________

8. Name two items that belong in a professional technical handoff.

________________________________________________________________________________________________

________________________________________________________________________________________________

9. A teammate is unsure whether a circuit is safe to touch. What is the correct next action?

________________________________________________________________________________________________

________________________________________________________________________________________________

10. In one sentence, explain why a correct component can still be wrong for a specific system.

________________________________________________________________________________________________

________________________________________________________________________________________________

# Application architecture reference

| Architecture | Utility role | Storage role | Typical decision signal |
| --- | --- | --- | --- |
| Grid-tied | Available and interconnected | Often none | Bill reduction is central; outage operation is not required |
| Hybrid | Available | Selected backup or operating goals | Grid benefits plus defined resilience needs |
| Off-grid | Unavailable | Carries loads outside solar hours | Remote loads must operate without utility support |
| PV-direct | Unavailable or irrelevant | Usually none | Load can operate only when solar power is available |

## Design order

1.  Define the job: what must the system accomplish?

2.  Define the load: what must run, at what power, and for how long?

3.  Read the site: solar access, shade, weather, space, access, and maintenance.

4.  Choose a first-pass architecture.

5.  Map the energy path and name required protection and isolation points.

6.  Record unknowns; verify them before sizing or building.

Safety boundary Today's work is design-only. Use paper component cards or de-energized components. Do not connect modules, batteries, controllers, inverters, or utility circuits.

# Afternoon design lab

Mission Choose the PV architecture that best matches each client's real job. Show the energy path, identify the dominant constraint, state the safety boundary, and deliver a two-minute handoff.

## Team structure

Work in three teams of two. Start with the assigned scenario, then rotate if time permits.

Roles: designer and verifier/recorder. Switch roles after each scenario.

Use component cards or de-energized components only. No electrical connections are permitted.

## Required deliverables

1.  Architecture choice: grid-tied, hybrid, off-grid, or PV-direct.

2.  A labeled energy-path diagram from source to load.

3.  The critical load or operating need.

4.  Three site or client constraints and the dominant constraint.

5.  Protection, isolation, or labeling boundaries that must be designed before construction.

6.  At least two unknowns that require verification.

7.  A two-minute handoff: scope, decision, evidence, safety boundary, and unresolved items.

Acceptance criteria Match the need. Show the path. Name the constraint. State the safety boundary. Record the unknowns.

# Scenario 1 - Remote trail counter

A 12 V electronic trail counter uses 8 W continuously.

No utility service is available.

The site has seasonal shade and can go five days between service visits.

The counter must collect data day and night.

## Design brief

1. Architecture choice and one-sentence reason

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

2. Critical load or operating need

________________________________________________________________________________________________

________________________________________________________________________________________________

3. Energy path - write components in order and draw arrows

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

4. Dominant constraint and supporting evidence

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

5. Two additional constraints

________________________________________________________________________________________________

________________________________________________________________________________________________

6. Required protection, isolation, and labeling boundaries

________________________________________________________________________________________________

________________________________________________________________________________________________

7. Unknowns to verify before sizing or building

________________________________________________________________________________________________

________________________________________________________________________________________________

8. Two-minute handoff notes

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

# Scenario 2 - Small shop

Utility service is available.

The owner wants lower electric bills.

During an outage, internet equipment and office lighting totaling about 400 W should operate for four hours.

Roof area is available but one section receives afternoon shade.

## Design brief

1. Architecture choice and one-sentence reason

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

2. Critical load or operating need

________________________________________________________________________________________________

________________________________________________________________________________________________

3. Energy path - write components in order and draw arrows

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

4. Dominant constraint and supporting evidence

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

5. Two additional constraints

________________________________________________________________________________________________

________________________________________________________________________________________________

6. Required protection, isolation, and labeling boundaries

________________________________________________________________________________________________

________________________________________________________________________________________________

7. Unknowns to verify before sizing or building

________________________________________________________________________________________________

________________________________________________________________________________________________

8. Two-minute handoff notes

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

# Scenario 3 - Daytime water pump

A 300 W DC pump fills an elevated tank.

No utility service is available.

The pump only needs to run when strong sunlight is available.

The tank provides water after sunset; battery storage is not requested.

## Design brief

1. Architecture choice and one-sentence reason

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

2. Critical load or operating need

________________________________________________________________________________________________

________________________________________________________________________________________________

3. Energy path - write components in order and draw arrows

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

4. Dominant constraint and supporting evidence

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

5. Two additional constraints

________________________________________________________________________________________________

________________________________________________________________________________________________

6. Required protection, isolation, and labeling boundaries

________________________________________________________________________________________________

________________________________________________________________________________________________

7. Unknowns to verify before sizing or building

________________________________________________________________________________________________

________________________________________________________________________________________________

8. Two-minute handoff notes

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

# Peer verification and exit ticket

## Peer verification checklist

☐ The architecture matches the client's stated operating need.

☐ The energy path includes every major required function.

☐ The dominant constraint is supported by scenario evidence.

☐ Protection and isolation are treated as design requirements.

☐ Unknowns are recorded instead of guessed.

☐ The handoff separates verified facts from open questions.

## Exit ticket

1. Which operating need most strongly determines architecture?

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

2. When is storage unnecessary?

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

3. What is one question you must verify before Day 2 site assessment?

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________