PV101 CTS Cohort 6 Day 2 Master Print Packet

Electricity Basics, Multimeter Practice, Series and Parallel

| Program | Cohort | Day | Date |
| --- | --- | --- | --- |
| PV101 | CTS Cohort 6 | Day 2 | 2026-07-07 |

Source control note: This packet is intentionally titled and dated for reuse. Do not leave generated curriculum unlabeled in chat history, because apparently civilization enjoys preventable chaos.

# Architecture of the Day

Theme: Solar is not magic. It is light becoming electrical behavior that we can measure, predict, and respect.

Core idea: Before students install parts, they need to understand what the system is trying to do.

Primary skill: Explain voltage, current, resistance, power, AC/DC, and basic series/parallel behavior using plain language.

Assessment: Meter practice sheet, quiz, labeled circuit/system sketch, and exit ticket.

Room tone: Calm, practical, low-drama, one task at a time.

# Learning Targets

Students explain voltage, current, resistance, and power in simple language.

Students identify the difference between AC and DC.

Students use a multimeter to safely measure DC voltage.

Students explain why PV modules can be energized when exposed to light.

Students predict what changes in series and parallel connections.

Students connect the basics to the Cohort 6 RV solar capstone.

# Materials

Digital multimeters with leads

AA, AAA, and 9V batteries

Small 12V battery or low-voltage training power supply

Small PV training panel or module

Low-voltage lamp, LED, or fan load

Switch, fuse holder, terminal strip, jumper wires

Whiteboard markers, pencils, student packets, quiz copies

PPE as appropriate for the work area

# Instructor Day Flow

| Time | Block | Instructor Action |
| --- | --- | --- |
| 10:00-10:15 | Arrival reset | Green/yellow/red check-in. Set a calm safety tone. Board: Solar work rewards patience. Guessing is not a method. |
| 10:15-10:35 | Day 1 review | PV = photovoltaic. Photo = light. Voltaic = electricity/voltage. Connect review to the RV solar capstone. |
| 10:35-11:05 | Electricity basics | Teach voltage, current, resistance, power, and Watts = Volts x Amps. Keep it concrete and example-based. |
| 11:05-11:20 | AC vs DC | Solar panels and batteries are DC. Homes and many appliances are AC. Inverters translate DC to AC. |
| 11:20-11:30 | Break | Move bodies. Reset attention. |
| 11:30-12:05 | Multimeter practice | Inspect leads, set meter, measure batteries and low-voltage sources. Do not freestyle current measurement. |
| 12:05-12:30 | PV light demo | Measure panel voltage in light, shade, tilted, and covered conditions. Have students predict first. |
| 12:30-1:00 | Lunch | Clean table reset before leaving. |
| 1:00-1:35 | Series and parallel | Series stacks voltage. Parallel stacks amperage. Use math examples. |
| 1:35-2:10 | Mini-build or circuit trace | Source -> fuse -> switch -> load -> return. Students label current path and test points. |
| 2:10-2:25 | RV capstone bridge | Start the load list. AC/DC, voltage, watts/amps, hours per day, notes. |
| 2:25-2:45 | Quiz | 10-question Day 2 quiz. |
| 2:45-2:55 | Exit ticket | Understanding, practice need, safety rule, RV load to verify. |
| 2:55-3:00 | Cleanup | Meters off, leads wrapped, tools returned, batteries stored, papers collected. |

# Board Notes / Anchor Language

| Term | Plain Meaning |
| --- | --- |
| PV | Photovoltaic: photo = light, voltaic = electricity/voltage. |
| Voltage | Electrical pressure. Unit: volts (V). |
| Current | Electrical flow. Unit: amps (A). |
| Resistance | Opposition to flow. Unit: ohms. |
| Power | Work being done. Unit: watts (W). |
| Formula | Watts = Volts x Amps |
| DC | Direct current. One direction. Solar panels and batteries. |
| AC | Alternating current. Changes direction. Wall outlets and many appliances. |
| Series | Voltage adds. Current stays about the same. |
| Parallel | Current adds. Voltage stays about the same. |

# Multimeter Safety Rules

Inspect the meter and leads before use.

Set the meter before touching the circuit.

Black lead goes in COM.

Red lead goes in the correct port for the measurement.

Voltage is measured across two points.

Current is measured in series, not across a battery.

Never short a battery with meter leads.

If unsure, stop and ask.

# Instructor Scripts

| Moment | Script |
| --- | --- |
| Opening | Today we are learning the electrical alphabet of solar: voltage, current, resistance, power, AC/DC, and how panels behave when wired together. |
| Meter demo | The meter only does what you told it to do. If you tell it something wrong, it may obey. Set the meter before touching the circuit. |
| Series/parallel | Series stacks voltage. Parallel stacks amperage. We choose based on the system, not vibes. |
| RV bridge | The RV may contain mystery wiring and hidden fuses. Our rule is verification over labels. |
| Closing | Today was the alphabet. Tomorrow we start making system sentences: loads, batteries, controllers, inverters, wire, and fuses. |

# Student Handout and Practice Sheet

## Big Idea

Solar PV turns light into DC electricity. We use measurements instead of guesses.

## Electricity Words

Voltage = pressure

Current = flow

Resistance = opposition

Power = work being done

## Units

Voltage = volts = V

Current = amps = A

Resistance = ohms

Power = watts = W

## Main Formula

Watts = Volts x Amps

Amps = Watts / Volts

Volts = Watts / Amps

## AC vs DC

DC = direct current: batteries, solar panels, many RV loads

AC = alternating current: wall outlets and many home appliances

## Series and Parallel

Series = voltage adds

Parallel = current adds

Memory line: Series stacks voltage. Parallel stacks amperage.

## PV System Path

Solar panel -> charge controller -> battery -> loads

Battery -> inverter -> AC loads when needed

## Meter Practice Record

| Object Tested | Meter Setting | Expected Reading | Actual Reading | What Did It Tell You? |
| --- | --- | --- | --- | --- |
| AA battery | DC volts |  |  |  |
| 9V battery | DC volts |  |  |  |
| 12V source | DC volts |  |  |  |
| Small PV panel in light | DC volts |  |  |  |
| Small PV panel shaded/covered | DC volts |  |  |  |
| Wire continuity | Continuity |  |  |  |

## RV Solar Load Starter List

| Load | AC/DC | Voltage | Watts or Amps | Notes |
| --- | --- | --- | --- | --- |
| Water pump | DC | 12V | Unknown | Verify hidden fuse and label |
| Interior lights | DC | 12V | Unknown | Count fixtures |
| Fan | DC | 12V | Unknown | Check label or measure |
| USB charging | DC | 5V/12V | Unknown | Adapter losses |
| Outlet loads | AC | 120V | Unknown | Inverter needed |

## Exit Ticket

One thing I understand better today:
____________________________________________________________________________________________

One thing I still want to practice:
____________________________________________________________________________________________

One safety rule I need to remember:
____________________________________________________________________________________________

One RV load we should verify before designing the system:
____________________________________________________________________________________________

# PV101 Day 2 Quiz

Name: ________________________________    Date: 7/7/2026

1. What does PV stand for?

A. Power Voltage

B. Photovoltaic

C. Panel Verification

D. Positive Value

Answer: ______________________________________________

2. Solar panels produce what kind of electricity?

A. AC

B. DC

C. Static only

D. None unless plugged into a wall

Answer: ______________________________________________

3. Match: Voltage, Current, Resistance, Power.

Voltage = electrical pressure

Current = electrical flow

Resistance = opposition to flow

Power = work being done

Answer: ______________________________________________

4. What formula gives watts?

A. Watts = Volts x Amps

B. Watts = Ohms / Volts

C. Watts = Amps + Volts

D. Watts = Resistance x Time

Answer: ______________________________________________

5. A 12V fan draws 3A. How many watts is it using?

Work: ________________________________________________

Answer: ______________________________________________

6. A 60W light is running on a 12V system. How many amps does it draw?

Work: ________________________________________________

Answer: ______________________________________________

7. In series wiring, what usually increases?

A. Voltage

B. Resistance only

C. Wire color

D. Nothing

Answer: ______________________________________________

8. In parallel wiring, what usually increases?

A. Voltage

B. Current

C. Sunlight

D. Panel size

Answer: ______________________________________________

9. True or false: A solar panel can produce voltage whenever light is hitting it.

Answer: ______________________________________________

10. Why should you inspect meter leads before using a multimeter?

Answer: ______________________________________________

# PV101 Day 2 Quiz Answer Key

1. What does PV stand for?
Answer: B

2. Solar panels produce what kind of electricity?
Answer: B

3. Match: Voltage, Current, Resistance, Power.
Answer: Voltage=pressure; Current=flow; Resistance=opposition; Power=work

4. What formula gives watts?
Answer: A

5. A 12V fan draws 3A. How many watts is it using?
Answer: 36W

6. A 60W light is running on a 12V system. How many amps does it draw?
Answer: 5A

7. In series wiring, what usually increases?
Answer: A

8. In parallel wiring, what usually increases?
Answer: B

9. True or false: A solar panel can produce voltage whenever light is hitting it.
Answer: True

10. Why should you inspect meter leads before using a multimeter?
Answer: Accept: damaged leads can give bad readings, short risk, or unsafe testing.

# Google Classroom Post

PV101 Day 2 - Electricity Basics + Meter Practice

Today we covered the foundation needed for solar work: voltage, current, resistance, watts, AC vs DC, safe multimeter use, and the difference between series and parallel wiring.

Main idea:

Solar panels produce DC electricity when exposed to light.

Voltage pushes.

Current flows.

Resistance limits.

Power does work.

Watts = Volts x Amps.

Hands-on work:

- Measured batteries and low-voltage sources with a multimeter

- Observed how light and shade affect a small solar panel

- Compared series and parallel wiring

- Started connecting electrical basics to the RV solar capstone

Exit ticket:

1. One thing I understand better today

2. One thing I still want to practice

3. One safety rule I need to remember

4. One RV load we should verify before designing the system