# PV102 Day 6 Afternoon Lab

Balance-of-System Block Diagram

PV102 | CTS Day 22 | Afternoon Lab
Power electronics set system boundaries. | Tuesday, August 11, 2026

# Mission

Teams build a labeled block diagram and defend every protection/isolation point.

# Safety boundary

This lab is training work. Do not energize or connect equipment outside the instructor-approved procedure. When in doubt, stop, isolate, verify, and ask.

# Scenario options

12 V training system with PV module, controller, LiFePO4 battery, inverter, DC load, AC load.

48 V bus system concept with hybrid inverter/charger, battery bank, PV strings, AC distribution.

Remote DC-only sensor system with no inverter and limited service visits.

# Procedure

Choose a scenario and place component cards in energy-path order.

Draw source, storage, conversion, and load boundaries.

Add conceptual OCPD/disconnect labels where energy could feed a fault or where service isolation is needed.

Mark manufacturer-instruction-required decisions.

Run a peer safety check: can someone tell what is energized, isolated, and unknown?

Handoff the diagram and unresolved decisions.

# Acceptance criteria

Match the real job.

Show the energy or work path.

Name the dominant constraint.

State the safety boundary.

Record unknowns instead of guessing.

Give a handoff another crew can use.

# Team handoff form

| Item | Response |
| --- | --- |
| Team members |  |
| Scenario |  |
| Primary decision |  |
| Evidence |  |
| Safety boundary |  |
| Unknowns |  |
| Next safest action |  |