RCS05 | Morning Quiz - Answer Key

CTS Cohort 6 | CTS Day 13 | 2026-07-27 | 10:00 AM-3:00 PM | 6 students

Advanced Electrical: Switch, Light, and Troubleshooting

1. What is an open circuit?

A broken or incomplete current path.

2. What conductor path is controlled by a basic switch?

The ungrounded or hot path to the load.

3. What should troubleshooting begin with?

A known expected path and verified source/energy state.

4. What does continuity indicate?

Whether an unbroken conductive path exists; it does not prove safe energized operation.

5. Why make one correction at a time?

To preserve cause-and-effect evidence.

6. What is a pigtail?

A short conductor used to connect a splice to a device or bonding point.

7. May students use continuity on an energized circuit?

No.

8. What is the first failed checkpoint?

The first point along the expected path where the observed result stops matching the expected result.

9. What capstone devices are required?

Two outlets, one switch, and one light point.

10. What completes a professional repair?

Correction, retest, documentation, and instructor approval.

# Scoring guidance

One point per item. Accept technically equivalent wording. Use errors to reteach before lab release. Safety-boundary errors must be corrected explicitly before the student begins the related task.