# PV102 Day 3 Student Packet

Load Profile and Energy Budget

PV102 | CTS Day 19 | Student Print Packet
Watts are moments. Watt-hours are work over time. | Wednesday, August 5, 2026

# Today's rule

Watts are moments. Watt-hours are work over time.

# Learning targets

Distinguish watts from watt-hours.

Calculate daily energy use from power and time.

Rank loads by necessity, runtime, surge risk, and design impact.

Explain how load decisions affect PV, battery, inverter, and budget.

# Morning quiz

1. A 60 W load runs for 3 hours. How much energy is used?

A. 20 Wh

B. 60 Wh

C. 180 Wh

D. 3 Wh

2. Watts measure:

A. Instant power

B. Total daily energy only

C. Wire length

D. Battery age

3. Watt-hours measure:

A. Energy over time

B. Compass direction

C. Module frame size

D. Fuse color

4. The load that often changes inverter selection is:

A. Sticky note

B. Surge or motor load

C. Pencil

D. Label maker

5. A good client load interview asks:

A. What must run, when, and for how long?

B. What color is the inverter?

C. How fast can we buy parts?

D. Which battery looks best?

6. Calculate: 12 V x 5 A = ___ W.

________________________________________________________________________________________________

________________________________________________________________________________________________

7. Why should optional loads be separated from essential loads?

________________________________________________________________________________________________

________________________________________________________________________________________________

8. Name one load unknown that must be verified before final sizing.

________________________________________________________________________________________________

________________________________________________________________________________________________

9. What happens to required battery capacity when runtime doubles?

________________________________________________________________________________________________

________________________________________________________________________________________________

10. Write the Day 3 rule in your own words.

________________________________________________________________________________________________

________________________________________________________________________________________________

# Reference: decision order

| Step | Action | Proof question |
| --- | --- | --- |
| 1 | Define the job | What must the system accomplish? |
| 2 | Collect evidence | What is measured, observed, or still unknown? |
| 3 | Choose boundary | What must be protected, isolated, sized, or verified? |
| 4 | Defend the choice | What evidence supports the decision? |
| 5 | Handoff | What should the next person know? |

# Client Load Interview and Energy Budget

Teams interview a mock client, build a daily load table, and recommend one load reduction.

## Scenario options

Tiny shop: lights, laptop, phone charging, modem/router, small fridge, occasional circular saw.

Remote trail counter: sensor, camera, cellular modem, winter operation, limited service visits.

Bus conversion: lighting, fridge, fan, water pump, laptop, phone charging, occasional induction cooktop.

## Team procedure

Choose one scenario and list all loads.

For each load, record watts, hours per day, daily Wh, priority, and unknowns.

Mark surge or motor loads.

Calculate total daily Wh and essential daily Wh.

Recommend one load change that improves system practicality without weakening the client need.

Prepare a handoff showing the load table and top three unknowns.

## Team worksheet

| Prompt | Team response |
| --- | --- |
| System/job statement |  |
| Evidence used |  |
| Decision made |  |
| Safety boundary |  |
| Top unknowns |  |
| Next action |  |

# Exit ticket

Which load drove the design hardest today: highest watts, longest runtime, or highest priority? Explain.

________________________________________________________________________________________________

________________________________________________________________________________________________