Afternoon Lab - Commissioning & Fault Finding

PV101 Day 5 - Monday, July 13, 2026

| Lab purpose | Use the completed Day 4 systems as commissioning and troubleshooting trainers. Students inspect, measure, diagnose, document, and restore. |
| --- | --- |
| Team setup | Five systems for six students. One two-person team is appropriate. Rotate roles so each student meters, documents, and explains evidence. |

# Roles

| Role | Job | Required statement |
| --- | --- | --- |
| Lead inspector | Traces path, checks labels, fuse placement, conductor condition. | I can point to every source and protection point. |
| Meter lead | Selects range/ports and takes required readings. | I know the expected reading before I measure. |
| Recorder / verifier | Records evidence, watches sequence, confirms restoration. | We change one thing and verify the result. |

# Part A - Commission the existing build

Do not dismantle the system. Begin with a visual inspection.

Draw or trace: PV -> controller -> battery -> fuse/protection -> inverter/load.

Circle every current source and star every protection point.

Check polarity labels, exposed copper, strain relief, fuse access, and serviceability.

Record battery open-circuit voltage: ______ V.

Record controller battery-terminal voltage: ______ V.

Record PV/input voltage when safe and available: ______ V.

Record load/inverter input voltage under load: ______ V.

Record load-end voltage under load: ______ V.

Voltage drop = source-end voltage - load-end voltage: ______ V.

Instructor initials for commissioned system: ______.

# Part B - Fault diagnosis

Fault card or station: ____________________________

Observed symptom: ______________________________________________________________

Expected normal condition: ______________________________________________________________

Prediction before measuring: ______________________________________________________________

First measurement and result: ______________________________________________________________

Second measurement and result: ______________________________________________________________

Fault identified: ______________________________________________________________

Single corrective action: ______________________________________________________________

Evidence system returned to normal: ______________________________________________________________

# Part C - Wire and fuse reasoning

Assigned or measured load power: ______ W.

System voltage under load: ______ V.

Estimated current, A = W / V: ______ A.

Installed or proposed conductor size: __________________.

Installed or proposed fuse size: __________________.

Explain why the fuse protects the conductor: ___________________________________

Would a longer wire run improve or worsen voltage drop? Explain: ________________________

# Part D - Serviceability handoff

Add or correct one useful label.

Identify one future troubleshooting test point.

Confirm fuse access and battery isolation method.

Leave a brief note for the next technician: ____________________________________________.

# Checkout rubric

| Skill | Score |
| --- | --- |
| Traces complete power path | 0 1 2 |
| Predicts expected reading before measuring | 0 1 2 |
| Uses meter safely and correctly | 0 1 2 |
| Diagnoses from evidence rather than guessing | 0 1 2 |
| Explains fuse and conductor relationship | 0 1 2 |
| Measures and explains voltage drop | 0 1 2 |
| Restores and verifies known-good condition | 0 1 2 |
| Improves labeling/serviceability | 0 1 2 |

# Exit ticket

One reading that proved something today: ______________________________

One fault I can now diagnose systematically: __________________________

One reason not to rebuild a working system just to practice: __________________