# PV102 Day 4 Student Packet

Module Behavior String Design and Shade

PV102 | CTS Day 20 | Student Print Packet
String design follows limits, temperature, and shade reality. | Thursday, August 6, 2026

# Today's rule

String design follows limits, temperature, and shade reality.

# Learning targets

Predict how series and parallel wiring affect PV string voltage and current.

Explain why controller input limits control string design.

Recognize shade as both a production and mismatch problem.

Use a safe paper/string model to compare design options before hardware work.

# Morning quiz

1. In series wiring, matching module voltage primarily:

A. Adds

B. Disappears

C. Becomes AC

D. Turns into resistance

2. In parallel wiring, matching branch current primarily:

A. Adds

B. Becomes zero

C. Changes to frequency

D. Removes voltage

3. The charge controller input limit is important because:

A. Exceeding limits can damage equipment or create unsafe conditions

B. It chooses wire color

C. It makes sunlight brighter

D. It stores energy

4. Shade on one module can affect:

A. Only the paper worksheet

B. Production and string behavior

C. Lunch time only

D. Nothing if the roof is new

5. Cold temperature generally makes PV voltage:

A. Higher

B. Always zero

C. AC only

D. Unrelated to design

6. Why should string design be checked before connecting equipment?

________________________________________________________________________________________________

________________________________________________________________________________________________

7. A controller allows 100 V max input. Should a design near 99 V be accepted without temperature check?

________________________________________________________________________________________________

________________________________________________________________________________________________

8. Name one reason parallel strings may require added protection.

________________________________________________________________________________________________

________________________________________________________________________________________________

9. What evidence from Day 2 matters most for shade design?

________________________________________________________________________________________________

________________________________________________________________________________________________

10. Write the Day 4 rule in your own words.

________________________________________________________________________________________________

________________________________________________________________________________________________

# Reference: decision order

| Step | Action | Proof question |
| --- | --- | --- |
| 1 | Define the job | What must the system accomplish? |
| 2 | Collect evidence | What is measured, observed, or still unknown? |
| 3 | Choose boundary | What must be protected, isolated, sized, or verified? |
| 4 | Defend the choice | What evidence supports the decision? |
| 5 | Handoff | What should the next person know? |

# String Design Paper Lab

Teams compare three paper string options against controller limits and shade constraints.

## Scenario options

Four 22 Vmp / 27 Voc training modules feeding a 100 V controller.

Two shaded morning modules and two unshaded modules on a shed roof.

Long ground-rack wire run where voltage drop and current both matter.

## Team procedure

Read the module and controller cards.

Build three possible string layouts using cards.

Calculate approximate operating voltage/current and open-circuit voltage.

Mark shade exposure from Day 2 scenario notes.

Reject any option that violates a limit or ignores shade evidence.

Handoff the safest first-pass string layout and its required verification checks.

## Team worksheet

| Prompt | Team response |
| --- | --- |
| System/job statement |  |
| Evidence used |  |
| Decision made |  |
| Safety boundary |  |
| Top unknowns |  |
| Next action |  |

# Exit ticket

What made a string option unacceptable today: equipment limit, shade reality, or missing information?

________________________________________________________________________________________________

________________________________________________________________________________________________