# PV101 Day 7 Student Print Packet

Compiled packet

Day_07_PV101_Load_Assessment_and_System_Sizing_2026-07-15

Contents follow in instructional order.

# PV101 Day 7 Morning Quiz

Student copy | 10-15 minutes

Day_07_PV101_Load_Assessment_and_System_Sizing_2026-07-15

Name: __________________________________  Date: __________________

Answer briefly. Show work and units where needed.

1. What does AHJ stand for in a permitting context?
__________________________________________________________________________________
__________________________________________________________________________________

2. Why should the AHJ and utility requirements be checked before an installation begins?
__________________________________________________________________________________
__________________________________________________________________________________

3. Why must an incentive be verified with a current authoritative source?
__________________________________________________________________________________
__________________________________________________________________________________

4. Name two useful authoritative resources for planning permitted PV work.
__________________________________________________________________________________
__________________________________________________________________________________

5. What is the purpose of a permit and inspection process?
__________________________________________________________________________________
__________________________________________________________________________________

6. What should you do first when a practice question includes units?
__________________________________________________________________________________
__________________________________________________________________________________

7. When two answers seem possible, what type of answer takes priority in PV work?
__________________________________________________________________________________
__________________________________________________________________________________

8. Why should a calculated answer be checked for reasonableness?
__________________________________________________________________________________
__________________________________________________________________________________

9. Why is reviewing a missed practice question useful?
__________________________________________________________________________________
__________________________________________________________________________________

10. State the course rule that replaces guessing.
__________________________________________________________________________________
__________________________________________________________________________________

# PV101 Day 7 Student Worksheet

Load assessment and basic system sizing | Guided practice

Day_07_PV101_Load_Assessment_and_System_Sizing_2026-07-15

Name: __________________________________  Date: __________________

# Operating rules

Use only the values provided.

Show formulas, numbers, and units.

Circle assumptions.

Check reasonableness before asking for signoff.

This is guided practice—not a product design or proposal.

# Provided training assumptions

| Item | Training value |
| --- | --- |
| System voltage | 12 V |
| Usable battery fraction / maximum depth of discharge | 80% for this exercise |
| Autonomy for this exercise | 1 day |
| Peak sun hours for this exercise | 4 h/day |
| Loss multiplier for this exercise | 1.25 |
| Inverter headroom for this exercise | 25% above concurrent AC load |

# Part A: RV training loads

| Load | Power | Hours/day | Calculation | Wh/day |
| --- | --- | --- | --- | --- |
| 3 interior LED lights | 8 W each | 4 h |  |  |
| Water pump | 60 W | 0.5 h |  |  |
| Vent fan | 36 W | 3 h |  |  |
| Phone/USB charging | 24 W | 2 h |  |  |
| Laptop charger (AC) | 65 W | 2 h |  |  |

Total daily energy: __________________ Wh/day

# Part B: Storage

Required stored Wh = daily Wh x autonomy = __________________________________
__________________________________________________________________________________

Battery nameplate Wh = required stored Wh / usable fraction = ________________________
__________________________________________________________________________________

Battery Ah at 12 V = battery nameplate Wh / 12 V = _________________________________
__________________________________________________________________________________

Reasonableness sentence: __________________________________________________________
__________________________________________________________________________________

# Part C: Array and inverter

Raw array watts = daily Wh / peak sun hours = __________________ W
__________________________________________________________________________________

Adjusted array watts = raw array watts x 1.25 = __________________ W
__________________________________________________________________________________

Concurrent AC load used for inverter sizing: __________________ W
__________________________________________________________________________________

Minimum inverter rating with 25% headroom: __________________ W
__________________________________________________________________________________

What load or assumption most affects the result? _____________________________________
__________________________________________________________________________________

# Part D: One-line diagram

Draw: PV -> controller -> battery -> inverter/DC loads. Show return paths, protection, disconnects, labels, and meter points.

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

# Verification and signoff

| Check | Peer | Instructor |
| --- | --- | --- |
| Loads and hours copied correctly |  |  |
| Every calculation shows units |  |  |
| Assumptions match provided values |  |  |
| Answers are reasonable and rounded clearly |  |  |
| Diagram includes both directions/return path |  |  |
| Protection, disconnects, labels, and test points shown |  |  |

# PV101 Day 7 Guided-Practice Guide

Student-facing

Day_07_PV101_Load_Assessment_and_System_Sizing_2026-07-15

Name: __________________________________  Date: __________________

# Purpose

Guide students through a bounded sizing exercise using only the provided RV training loads and assumptions.

# Tools and materials

Student worksheet

Calculator and pencil

Ruler

Provided training assumptions

No physical PV equipment required

# Procedure

Complete the loads table.

Checkpoint 1: instructor verifies daily Wh.

Calculate storage using the provided autonomy and 80% usable-fraction / maximum-depth-of-discharge assumption.

Checkpoint 2: instructor verifies units and arithmetic.

Calculate raw and adjusted array watts.

Select inverter headroom from concurrent AC load.

Draw and peer-check the one-line diagram.

Revise and obtain instructor signoff.

# Common mistakes

Adding watts without multiplying by hours

Confusing W with Wh

Dividing by the wrong voltage

Omitting the usable-fraction / depth-of-discharge adjustment

Sizing inverter from daily energy instead of simultaneous power

Inventing equipment values not supplied

# Cleanup

Return calculators and rulers.

Submit worksheet and reflection.

Clear tables and recycle scratch paper as directed.

# PV101 Day 7 Reflection Questions

Student copy

Day_07_PV101_Load_Assessment_and_System_Sizing_2026-07-15

Name: __________________________________  Date: __________________

1. What sizing step makes the most sense to you now?
__________________________________________________________________________________
__________________________________________________________________________________

2. Where are you most likely to make a unit or assumption error?
__________________________________________________________________________________
__________________________________________________________________________________

3. How does verification change a design calculation?
__________________________________________________________________________________
__________________________________________________________________________________

4. What did Days 1-6 teach that helped today?
__________________________________________________________________________________
__________________________________________________________________________________

5. What would you check before trusting someone else’s sizing worksheet?
__________________________________________________________________________________
__________________________________________________________________________________