# Day 38 EBIKE101 Afternoon Lab — Homunculous Finish-Work Diagnosis

Program: CTS Cohort 6

Course: EBIKE101

Date: Tuesday, 2026-09-08

Lab type: Live project diagnosis and approved finish work

## Lab objective

Students will diagnose the current state of the E-bike Homunculous, identify what is required to finish it, and begin only the safe work that has been verified and approved.

## Instructor setup

Before students begin:

1. Place the bike where all sides can be accessed.

2. Keep battery connection under instructor control.

3. Remove or isolate any obviously unsafe condition.

4. Stage hand tools, meter, labels, zip ties, and worksheet.

5. Define the boundary: diagnosis first, power later only if earned by evidence.

## Lab roles

Assign or rotate:

Mechanical lead: checks frame, wheels, brakes, fasteners, chain path.

Electrical tracer: maps source, protection, controller, controls, motor.

Documentation lead: worksheet, board, photo evidence, parts list.

Safety observer: watches pinch points, battery boundary, tool use, moving parts.

Parts lead: organizes missing parts, hardware, adapters, labels.

## Part 1 — Mechanical readiness inspection

Students inspect:

Wheel security

Axle nut condition

Torque arm or anti-rotation support if applicable

Brake function

Tire condition and pressure

Chain alignment

Crank/pedal condition

Sharp edges

Cable rub points

Loose fasteners

Battery/controller mounting

Stop if:

Motor wheel can shift in dropout

Brakes cannot stop the wheel

Chain path is unsafe

Battery mount is loose

Any wire can reach tire, chain, brake rotor, or crank

## Part 2 — Electrical/control inventory

Students identify and document:

Battery voltage/rating if labeled

Controller label/rating if visible

Main power connector

Fuse/protection location

Display connector

PAS connector/sensor/magnet ring

Throttle presence or absence

Brake cutoff connectors

Motor cable

Accessory wires

Damaged connectors

Unknown wires

Rule:

Unknown wires stay unknown until traced, labeled, or verified. Guessing does not become truth just because someone says it confidently in a shop voice.

## Part 3 — Diagnostic decision tree

Use this order:

1. Is the bike mechanically safe to inspect?

2. Is the battery appropriate and controlled?

3. Is there protection between battery and controller?

4. Does the controller have a secure mount?

5. Are control inputs present?

6. Are brake cutoffs present and not stuck active?

7. Is the PAS sensor installed and aligned?

8. Is the throttle present, missing, or optional for this controller?

9. Is the motor cable undamaged and supported?

10. What must be fixed before first power-up?

## Part 4 — Approved finish work

Allowed student work may include:

Brake adjustment

Tightening non-critical hardware under supervision

Labeling wires

Improving cable routing

Adding strain relief

Cleaning mounting surfaces

Organizing connectors

Creating a parts-needed list

Documenting evidence

Non-powered meter checks where appropriate

Instructor-only or instructor-controlled work:

Battery connection

First power-up

Controller testing

Motor spin testing

Fuse replacement if cause is unknown

Any modification to high-current wiring

Any ride test

## Part 5 — First power-up gate

Do not perform a powered test unless all are checked:

Mechanical assembly secure

Brakes functional

Drive wheel controlled

Battery voltage/rating appropriate

Fuse/protection path verified

Controller mounted

No melted/damaged wires

No unrestrained motor cables

No hands near moving parts

Instructor gives explicit approval

## Completion target

The lab is complete when the team can answer:

1. What works or appears complete?

2. What is missing?

3. What is unsafe?

4. What is unknown?

5. What should be done next?

Finishing the bike is excellent. Finishing the diagnosis is required.