# PV101 Day 8 Morning Quiz Answer Key

Instructor copy

Day_08_PV101_Final_Review_and_Assessment_2026-07-16

1. A 24 V training load draws 2 A. What power does it use?
48 W; W = V x A = 24 x 2.

2. A 40 W device operates for 2 hours. What daily energy does it use?
80 Wh; Wh = W x h = 40 x 2.

3. Before testing continuity across a fuse, name the required energy state and meter function.
Circuit de-energized and verified; meter on continuity/ohms with correct leads and ports.

4. A wire label says polarity is correct. What evidence should verify the label?
A correctly configured voltage measurement at documented points with the observed polarity compared to the expected polarity.

5. Why write the expected value and unit before taking a measurement?
It helps select the correct function/range and makes the observed result useful evidence instead of an unexplained number.