PV101 Day 06 Morning Quiz

Troubleshooting, fuses, and meter habits

| Course | PV101 |
| --- | --- |
| Cohort | CTS Cohort 6 |
| Date | Tuesday, July 14, 2026 |
| Theme | Troubleshooting and Hidden Fuses |

Name: ________________________________   Team: ____________________

Directions: Choose the best answer.

1. What is the job of a fuse in a PV circuit?

A. To protect the wire and equipment by opening during too much current

B. To increase voltage

C. To store energy

D. To convert DC to AC

2. A circuit shows voltage on one side of a fuse but not the other. What is the most likely issue?

A. The inverter is oversized

B. The fuse is open or the fuse holder connection is bad

C. The battery is fully charged

D. The module is shaded

3. Before replacing an expensive part, what should a technician do?

A. Guess quickly

B. Verify power, connections, and protection devices with measurements

C. Bypass all protection

D. Change the battery first

4. Continuity mode is best used when the circuit is:

A. Energized

B. De-energized

C. Connected to the PV module in full sun

D. Running the inverter

5. If a fuse is blown, what must be considered before replacing it?

A. Only the color of the fuse

B. Why it opened and whether the replacement rating is correct

C. Whether the inverter is loud

D. The brand of the meter

6. A good troubleshooting order is:

A. Replace parts, then measure

B. Follow power flow and verify at each point

C. Skip the wiring diagram

D. Ask another team

7. What meter setting checks for DC voltage in a 12V battery system?

A. DC volts

B. AC amps

C. Resistance on an energized circuit

D. Temperature

8. Which statement is most professional?

A. It has to be the controller

B. I proved voltage stops at the fuse output

C. I changed parts until it worked

D. The battery looks fine

9. Why should a fuse never be bypassed as a repair?

A. It makes the system too quiet

B. It removes protection and can create a fire or equipment hazard

C. It lowers the battery voltage

D. It changes sunlight into AC

10. Today's main lesson is:

A. Big parts always fail first

B. The cheapest part can stop the whole system

C. Meters are optional

D. Fuses are only labels