# PV101 Day 8 Student Worksheet Answer Key

Instructor copy

Day_08_PV101_Final_Review_and_Assessment_2026-07-16

# Formula and units check

Power formula: W = V x A.

24 W x 3 h = 72 Wh.

0.5 kW x 1,000 W/kW = 500 W.

In series, voltage adds. In matched parallel paths, current adds.

# Verification chain

Symptom -> test -> evidence -> conclusion. Accept equivalent wording that keeps the observation before the test and the conclusion after evidence.

# System path

Accept PV -> controller -> battery -> protected inverter/DC load path with a complete return, one source-side fuse, one disconnect, and one logical meter point.

# Misconceptions

| Statement | Answer | Correction |
| --- | --- | --- |
| Wire color proves polarity. | Incorrect | Color is a clue; verify polarity with labels, tracing, and the correct meter setup. |
| Continuity is tested on an energized circuit. | Incorrect | Continuity/ohms testing requires the circuit to be de-energized. |
| Replacing a fuse proves the fault is repaired. | Incorrect | Find the cause and verify the restored known-good state. |
| Loads are the starting point for sizing. | Correct | Document watts, hours, simultaneous loads, and assumptions before sizing. |
| Cleanup is part of technical work. | Correct | A professional closeout includes safe state, material count, records, and room reset. |